Instructions

Honor system pledge: I have neither given nor received any aid while working on this examination.  I have not used a calculator nor referenced any material while working on this examination. I ceased work when instructed to do so.

Printed Name: Soham Gandhi

Read and follow the instructions carefully.

- Print your NAME to sign the pledge on this page.

- You may not receive help from anyone. You must adhere to the Virginia Tech Honor Code.

- This is an open book examination. You may use the MIPS Reference Sheet included with this exam.

- The exam consists of 4 problems as well as the MIPS Reference Sheet). Be sure that all these pages are present in your copy.

- If you need more space, use any additional page as appropriate, but clearly indicate when you have done this.

- Clearly show how you arrive at each answer, and clearly indicate your final answers. Partial credit is possible on many problems.

- If a question seems ambiguous, give your reasons and state your assumptions clearly. Using faulty assumptions to simplify a problem will result in deductions.

- You can get exam clarification by making Piazza posts. For any questions that may contain ideas to solve the exam problems, make the post private. Be aware that public posts are a potential Honor code violation.

- To submit your exam, type up the solutions or scan the written work and upload on Canvas. It is your responsibility to make sure that the scanned images are readable and uploaded before the deadline.

| Problem | Points |
| --- | --- |
| 1 | / 40 |
| 2 | / 20 |
| 3 | / 20 |
| 4 | / 20 |
| Total | / 100 |

# Question #1: MIPS Instructions [40 Points]

## [20 points] Translate the following C code into assembly. Assume that the variables i and j are stored in $s0 and $s1, respectively. The base addresses of the arrays A and B are stored in $s2 and $s3, respectively. Assume i and j are positive integers, and the arrays A and B contain integer elements (4 bytes each).

## A[i] = B[j] - 2;

## // Address spaced out 4 bits therefore must shift left twice

## sll $s0, $s0, 2

## sll $s1, $s1, 2

## add $s2, $s0	// Address + offset for A[i]

## add $s3, $s1	// Address + offset for B[j]

## lw $t1, ($s3)	// B[j] value

## addi $t2, $t1, -2

## sw $t2, ($s2)

## [20 points] Translate the following C code to MIPS assembly code. Use a

## minimum number of instructions. Assume that the values of d, i, and n are in

## registers $s0, $t0, and $t1, respectively. Also, assume that register $s1 holds

## the base address of the array D.

d = 0;

for (i = 0; i < n; i++)

d = d + D[i];

d = d * 2;

and $s0, $s0, $zero	// d = 0;

and $t0, $t0, $zero 	// i =0;

addi $s1, $s1, 4		// Next address for D

lw $t2, ($s1)		// get D[i]

add $s0, $s0, $t2	// d = d + D[i];

addi $t0, $t0, 1		// i++;

bne $t0, $t1, -4 		// i < n

sll $s0, $s0, 1		// d = d * 2

Question #2: Assembly [20 Points]

We would like to translate the following MIPS instructions into 32-bit machine code by filling in the table for each instruction. If a particular field is not applicable, you can put “N/A” in that field. Either decimal or hexadecimal (or a mix of both) are acceptable. The first instruction is done for your reference. ** Do not worry about what the code does, only its representation in machine code. **

| LOOP: | slt | $s2, $s0, $s1 |
| --- | --- | --- |
|  | beq | $s2, $0, DONE |
|  | add | $t0, $t0, $s3 |
| DONE: | ... |  |

Note that each column in the table indicates the valid instruction format(s) for that field.

| Instruction | Format (R/I/J) | OP (R/I/J) | RS (R/I) | RT (R/I) | RD (R) | SHAMT (R) | FUNCT (R) | Immediate (I) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| slt $s2, $s0, $s1 | R | 0 | 16 | 17 | 18 | 0 | 0x2A | N/A |
| beq $s2, $0, DONE | I | 4 | 16 | 0 | N/A | N/A | N/A | 2 |
| add $t0, $t0, $s3 | R | 0 | 8 | 19 | 8 | 0(N/A) | 0x20 | N/A |

Question #3: Disassembly [20 Points]

Consider the following two MIPS instructions encoded in 32-bit machine code.

(1) 0x8fa40004

(2) 0x00240821

- [8 points] Write down the opcode for each instruction in binary and identify the instruction type (R, I, J)

- 0x8f  1000 1111  Opcode: 10 0011: 0x23  lw type I

- 0x00 R type  funct: 11 0001: 0x21 addu type R

- [12 points] Write down two different versions of the MIPS assembly instruction using both register number (e.g., $1) and register name (e.g., $1 has the name $at) for the same two instructions.

(1) 0x8fa40004

Using register number:

1000 1111 1010 0100 0000 0000 0000 0100

100011 11101 00100 0000 0000 0000 0100

Rs: $29

Rt: $4

lw $4 4($29)

Using register name:

lw $a0 4($sp)

(2) 0x00240821

Using register number:

0000 0000 0010 0100 0000 1000 0010 0001

000000 00001 00100 00001 00000 100001

Rs: $1

Rt: $4

Rd: $1

addu $1, $1, $4

Using register name:

addu $at, $at, $a0

Question #4: The Stack [20 Points]

Consider the following lines of code that represent a MIPS procedure func, called from main. For this question, we do not care about the functionality of func or main. Our goal is to make sure both func (next page) and main (below) follow the conventions of MIPS procedure calls. Specifically, use the stack to follow the caller / callee conventions for saving registers and to allocate / deallocate stack frames and their standard contents. Assume that func uses $a0 as input and returns the output in $v0, and that func itself calls another function that saves registers appropriately. Add as many lines of MIPS code as you need to complete both functions. Do not put unnecessary data on the stack. There may be more than one correct answer. Use the provided spaces to insert your MIPS instructions.

## Some spaces may be left empty.

main:	...

li	$a0, 5

add	$t0, $a0, $s0

// preserve $t0

addi $sp, $sp, -4

sw $t0, 0($sp)

jal	func

lw $t0, 0($sp)

addi $sp, $sp, 4

add	$t0, $t0, $v0 add	$t0, $t0, $s0

...

Question #4 (continued): The Stack [20 Points]

func:

// preserve s0

addi $sp, $sp, -4

sw $s0, 0($sp)

li	$s0, 5

add	$t0, $a0, $s0

// No need to preserve t0 since it isn’t used later

... (another  function  is  called here)

add	$v0, $a0, $v0

lw $s0, 0($sp)

addi $sp, $sp, 4

jr	$ra